### STANDARD OPERATING PROCEDURE

### Preparing for Application of

### Isocyanate / Water / Solvent Base Paint Products

### DO NOT USE UNLESS YOU ARE TRAINED, COMPETENT & HAVE PERMISSION

### What warnings and/or protective measures must be taken when using this equipment?

### EQUIPMENT REQUIRED FOR THIS TASK…                                                                                         ….    .…

Spraying equipment

### SAFE WORK PROCEDURE ………                                                             ………….… ………………   ………… …………..

List actions to be taken and order.

| 1 | Ensure all PPE is correctly fitted and in good working condition. |
| --- | --- |
| 2 | Unroll airline from storage holder, ensuring there are no kinks in air hose. |
| 3 | When in designated area for spraying, attach air hose to air fed mask belt with coupling by holding the filter component (attached to belt) and pushing firmly in an upward direction to ensure the connection has been made. This will be identified by a clicking sound and a tight seal with no air leaks. |
| 4 | Attach spray gun to air fed belt airline by pushing firmly with spray gun and airline until connection is made. Correct connection is identified by the airline being attached to spray gun body with a clicking sound and a tight seal with no air leaks. |
| 5 | When spray gun has made a proper seal, aim spray gun at object and squeeze trigger until paint comes from front of spray gun. As paint disperses from spray gun move the spray gun over the desired object until application is complete. |
| 6 | Use spray gun as per manufacturer’s recommendations. |
| 7 | To disconnect spray gun from airline pull down on outer part of coupling. This outer part of the coupling will move downward allowing the spray gun to be disconnected from airline. Correct disconnection will be identified by the sound of air being released. |
| 8 | To disconnect airline from air fed mask, push the green button on coupling and pull down, this will release the airline. |

This Standard Operating Procedure does not necessarily cover all possible hazards associated with the equipment. It should be used in conjunction with other references. It is designed as an adjunct to training and to act as a reminder to trained, competent operators.

| ASSESSMENT DATE: / / IF USED AS RECORD OF COMPENTENCY | Verified by Supervisor: |  | (name & signature) |
| --- | --- | --- | --- |
|  | Competent operator: |  | (name & signature) |

Version 1